[image: image1.jpg]BOARD

=3

o)



                TIMING CHECKLIST

Interconnects
1.
Is there wire? Does it work? Are you sure?

Equipment
2.
Is the equipment on the current list of timing equipment 

homologated by the FIS Timing Working Group? 


(fis-ski.com/DB/information/homologated-timing-equipment.html)

Setup
3.  Are the Time-of-Day times immediately and automatically sequentially recorded on printing strips or memorized electronically at the maximum precision of the timing device according to the requirements for homologation? (Refer to the above website to verify the required precision for each device.) 
Software
4.
If timing software is used to calculate times, is it calculating net 



times using the precision of ToD as used in the timing device?   

Hand Timing
5.
Is hand timing that records to at least 1/100th (.01) precision available?

Quality
6.
Is the timing crew aware that hand times, even if not used, should 

be within acceptable limits?

Start Gate
7.
Does the Start Gate meet the following requirements?



a.
Does the start have 2 separate electronically isolated switch contacts for triggering the start inputs from both System A and System B?


b.
One wand?



c.
Installed at proper height?



d.
Are identical Start Gate mechanism & Wand available for replacement?



e.
Is the placement marked on the Start Posts?



f.
Are the Start Posts solidly installed?

Finish Eyes
8.
Do the Finish Photocells meet the following requirements?



a.
Are 2 sets available?



b.
Are 4 posts with a maximum diameter of 6cm available for mounting photocells? (2 are acceptable; replacements must be available)


c.
Is the vertical separation between the beams no greater than 20cm?

Synchronization
9.
Have system A and System B been synchronized as close as possible to 
the scheduled start time for the first run of the day?


10.
Has the synchronization been performed from one single contact switch for 
all timing devices?

Systems

11.
If System A fails, is the timing crew aware of the requirement to use System B times to calculate a replacement time (EET) for the missing impulse?  

12.
Is the timing crew aware that manual timekeeping is not used to calculate a 
replacement time (EET) impulse unless both System A and System B fail?  
Season 2026

